| CIVIL AVIATION DIRECTORATE Malta Transport Centre, Pantar Road, Lija LJA 2021, Malta Tel: +356 21222203 Fax: +356 22555 5634 e-mail: civil.aviation@transport.gov.mt, url: www.transport.gov.mt |  |
| --- | --- |

Part-26 – Additional Airworthiness Specifications for Operations

Subpart B - Large Aeroplanes

Compliance Checklist

| Aircraft Registration: |  |
| --- | --- |
| Aircraft Type : |  |
| Serial Number: |  |
| Year of Manufacture : |  |
| Maximum Certificated Take-Off Mass (MCTOM): |  |
| Maximum Operational Passenger Seating Configuration (MOPSC): |  |
| First Individual CofA Date: |  |
| Maximum Operating Pressure Altitude (ft): |  |
| First Type Certification date: |  |

| Requirement |  | Subject | Compliant / X / N/A | Method of compliance or Reason for Non applicability |  |
| --- | --- | --- | --- | --- | --- |
| # 26.50- Seats, berths, safety belts, and harnesses |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport, type certified on or after 1 January 1958, shall ensure that each flight or cabin crew member seat and its restraint system are configured in order to provide an optimum level of protection in an emergency landing whilst allowing the occupant's necessary functions and facilitating rapid egress. (For compliance, refer to CS 26.50) | X N/A |  |  |
| # 26.60 - Emergency landing — dynamic conditions |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes used in commercial air transport of passengers, type-certified on or after 1 January 1958, and for which the individual certificate of airworthiness is first issued on or after 18 February 2021 shall demonstrate for each seat type design approved for occupancy during taxiing, take-off or landing that the occupant is protected when exposed to loads resulting from emergency landing conditions. The demonstration shall be made by one of the following means: (a) successfully completed dynamic tests; (b) rational analysis providing equivalent safety, based on dynamic tests of a similar seat type design. The obligation set out in the first paragraph shall not apply to the following seats: (a) flight deck crew seats, (b) seats in low-occupancy aeroplanes involved only in on-demand non-scheduled commercial air transport operations.’; (For compliance, refer to CS 26.60) | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| # 26.100 - Location of emergency exits |  |  |  |  |  |
|  |  | Operators of large aeroplanes that are used in commercial air transport and that have a maximum operational passenger seating configuration of more than 19 with one or more emergency exits deactivated shall ensure that the distance(s) between the remaining exits remains (remain) compatible with effective evacuation, except for large aeroplanes that have an emergency exit configuration installed and approved before 1 April 1999. (For compliance, refer to CS 26.100) | X N/A |  |  |
| # 26.105 - Emergency exit access |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport shall provide means to facilitate the rapid and easy movement of each passenger from their seat to any of the emergency exits in case of an emergency evacuation (For compliance, refer to CS 26.105) | X N/A |  |  |
| # 26.110 - Emergency exit markings |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes used in commercial air transport shall comply with the following: (a) means shall be provided to facilitate the location, access, and operation of emergency exits by cabin occupants under foreseeable conditions in the cabin in case of an emergency evacuation; (b) means shall be provided to facilitate the location and operation of emergency exits by personnel on the outside of the aeroplane in case of an emergency evacuation. (For compliance, refer to CS 26.110) | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| # 26.120 - Interior emergency lighting and emergency light operation |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport shall provide means to ensure that illuminated exit signage, general cabin and exit area illumination, and low level exit path illumination is available to facilitate the location of exits and movement of passengers to the exits in case of emergency evacuation. (For compliance, refer to CS 26.120) | X N/A |  |  |
| # 26.150 - Compartment interiors |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes used in commercial air transport shall comply with the following: (a) all materials and equipment used in compartments occupied by the crew or passengers shall demonstrate flammability characteristics compatible with minimising the effects of in-flight fires and the maintenance of survivable conditions in the cabin for a time commensurate with that needed to evacuate the aircraft; (b) smoking prohibition shall be indicated with placards; (c) disposal receptacles shall be such that containment of an internal fire is ensured; such receptacles shall be marked to prohibit the disposal of smoking materials. For compliance, refer to CS 26.150 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| (c) |  |  | X N/A |  |  |
| # 26.155 - Flammability of cargo compartment liners |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport, type certified after 1 January 1958, shall ensure that the liners of Class C or Class D cargo compartments are constructed of materials that adequately prevent the effects of a fire in the compartment from endangering the aircraft or its occupants. For compliance, refer to CS 26.155 | X N/A |  |  |
| # 26.156 - Thermal or acoustic insulation materials |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes used in commercial air transport, type certified on or after 1 January 1958, shall ensure that: (a) for large aeroplanes for which the first individual certificate of airworthiness was issued before 18 February 2021, when new thermal or acoustic insulation materials are installed as replacements on or after 18 February 2021, those new materials have flame propagation resistance characteristics which prevent or reduce the risk of flame propagation in the aeroplane; (b) for large aeroplanes for which the first individual certificate of airworthiness was issued on or after 18 February 2021, thermal and acoustic insulation materials have flame propagation resistance characteristics which prevent or reduce the risk of flame propagation in the aeroplane; (c) for large aeroplanes for which the first individual certificate of airworthiness was issued on or after 18 February 2021and with a passenger capacity of 20 or more, thermal and acoustic insulation materials (including the means of fastening the materials to the fuselage) installed in the lower half of the aeroplane have flame penetration resistance characteristics which prevent or reduce the risk of flame penetration into the aeroplane after an accident and which ensure survivable conditions in the cabin for the time needed to evacuate the aeroplane.’; For compliance, refer to CS 26.156 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| (c) |  |  | X N/A |  |  |
| # 26.157 Conversion of Class D compartments |  |  |  |  |  |
| # (a) | Operators of large aeroplanes that are used in commercial air transport, and that were type certified on or after 1 January 1958,shall ensure that: (a)for large aeroplanes, the operation of which involves the transport of passengers, each Class D cargo or baggage compartment, regardless of its volume, complies with the certification specifications applicable to a Class C compartment; (b)for large aeroplanes, the operation of which involves the transport of cargo only, each Class D cargo compartment, regardless of its volume, complies with the certification specifications applicable to either a Class C or a Class E compartment; This point shall not apply to operators of an aeroplane model listed in Table A.1 of Appendix 1 to this Annex.’; For compliance, refer to CS 26.157 |  | # X N/A |  |  |
| # (b) |  |  | # X N/A |  |  |
| # 26.160 - Lavatory fire protection |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes used in commercial air transport with a maximum operational passenger seating configuration of more than 19 shall comply with the following: Lavatories shall be equipped with: (a) smoke detection means; (b) means to automatically extinguish a fire occurring in each disposal receptacle. For compliance, refer to CS 26.160 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| # 26.170 - Fire extinguishers |  |  |  |  |  |
| (a) |  | Operators of large aeroplanes shall ensure that the following extinguishers do not use halon as an extinguishing agent: (a) built-in fire extinguishers for each lavatory waste receptacle for towels, paper or waste in large aeroplanes for which the first individual certificate of airworthiness is issued on or after 18 February 2020; (b) portable fire extinguishers in large aeroplanes for which the first individual certificate of airworthiness is issued on or after 18 May 2019.’; For compliance, refer to CS 26.170 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| # 26.200 - Landing gear aural warning |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport shall ensure that an appropriate landing gear aural warning device is installed in order to significantly reduce the likelihood of landings with landing gear inadvertently retracted. For compliance, refer to CS 26.200 | X N/A |  |  |
| # 26.201 Tyre inflation pressure |  |  |  |  |  |
|  | # Operators of large aeroplanes shall minimise the risk of a tyre being below its minimum serviceable inflation pressure during operation. For compliance, refer to CS 26.201 |  | # X N/A |  |  |
| # 26.205 Runway overrun awareness and alerting systems |  |  |  |  |  |
| # (a) | (a) Operators of large aeroplanes used in commercial air transport shall ensure that every large aeroplane for which the first individual certificate of airworthiness was issued on or after 1 July 2026, is equipped with a runway overrun awareness and alerting system. # (b) This system shall be designed in a manner allowing to reduce the risk of a longitudinal runway excursion during landing by providing an alert, in-flight and on the ground, to the flight crew when the aeroplane is at risk of not being able to stop within the available distance to the end of the runway. For compliance, refer to CS 26.205 |  | X N/A |  |  |
| # (b) |  |  | # X N/A |  |  |
| # 26.250 - Flight crew compartment door operating systems — single incapacitation |  |  |  |  |  |
|  |  | Operators of large aeroplanes used in commercial air transport shall ensure that flight crew compartment door operating systems, where installed, be provided with alternate opening means in order to facilitate access by cabin crew members into the flight crew compartment in the case of a single flight crew member incapacitation. For compliance, refer to CS 26.250 | X N/A |  |  |
| 26.334 Damage tolerance data for STCs and other existing major changes and repairs affecting those STCs or changes approved before 1 September 2003 |  |  |  |  |  |
| (a) |  | (a)Upon request of operators and organisations responsible for the management of continuing airworthiness, to support the compliance with point 26.370(a)(ii), for a change approval that was issued before 1 September 2003, the approval holder referred to in point 26.330(a) shall: (i)for changes and published repairs identified in accordance with points (a)(i) and (a)(iii) of point 26.332, respectively, perform a damage tolerance evaluation; (ii)establish and document the associated DTI, unless this has already been done. (b)The approval holder referred to in point (a) shall submit the damage tolerance data resulting from the evaluation performed in accordance with point (a)(i) to the Agency: (i)within 24 months from receipt of a request, for requests received before 26 February 2023, for approval; or (ii)before 26 February 2025or within 12 months from receipt of a request, whichever occurs later, for requests received on or after 26 February 2023, for approval. For compliance, refer to CS 26.334 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| 26.370 Maintenance programme |  |  |  |  |  |
| (a) |  | (a) Operators of large aeroplanes certified on or after 1 January 1958shall ensure that the maintenance programmes of those aeroplanes include: (i) for large aeroplanes certified to carry 30 passengers or more, or with a payload capacity greater than 3 402kg (7 500lb), approved DTIs; (ii)for large aeroplanes operated in accordance with Annex IV (Part-CAT) to Regulation (EU) No 965/2012 and certified to carry 30 passengers or more, or with a payload capacity greater than 3 402kg (7 500lb), a means for addressing the adverse effects that repairs and changes may have on fatigue-critical structures and on inspections provided for in point (a)(i); (iii)for large aeroplanes certified with a MTOW greater than 34 019kg (75 000lb), an approved LOV; (iv)a CPCP. (b)The following deadlines shall apply to the obligation referred to in point (a): (i)the maintenance programme of the aeroplanes shall be revised to address the requirements of points (a)(i), (ii) and (iv) before 26 February 2024or before operating the aeroplane, whichever occurs later; (ii)the maintenance programme of the aeroplanes shall be revised to address the requirements of point (a)(iii) before 26 August 2021, or 6 months after the publication of the LOV, or before operating the aeroplane, whichever occurs later. (c)For a large aeroplane model first certified before 26 February 2021and: (i)that is not operated anymore after 26 February 2024, points (a)(i), (a)(ii) and (a)(iv) shall not apply; (ii)that is not operated anymore after 26 August 2021, point (a)(iii) shall not apply; (iii)with a restricted TC issued before 26 February 2021in accordance with damage tolerance requirements, provided that it is not operated beyond 75 % of its design service goal and is primarily operated in support of the approval holder’s manufacturing operation, points (a)(i), (a)(ii) and (a)(iv) shall not apply. (d)For a large aeroplane model with a restricted TC issued before 26 February 2021and the primary purpose of which is firefighting, points (a)(i) and (a)(ii) shall not apply. For compliance, refer to CS 26.370 | X N/A |  |  |
| (b) |  |  | X N/A |  |  |
| (c) |  |  | X N/A |  |  |
| (d) |  |  | X N/A |  |  |
| NOTES : All sections of the compliance checklist shall be filled in by the operator. When the operator is filling the ‘Compliant’ Column of the checklist, it is intended that the equipment is installed on the aircraft and that the equipment conforms to the Acceptable Means of Compliance and Guidance Material section. The fourth column is intended to be filled with the method of compliance details. |  |  |  |  |  |

| Compliance Declaration Statement The Part-26 System & Equipment Requirements have been reviewed, and I certify that the above Part-26 Compliance Statement is a true reflection of the equipment installed in the subject aircraft. |  |
| --- | --- |
| Name: | Organisation: |
| Position: | Approval Reference: |
| Signature: | Date: |

Data Protection Notice

All data collected in this form is processed in accordance with the Privacy Laws that include General Data Protection Regulation (Regulation 2016/679/EU) and Chapter 440 of the Laws of Malta (Data Protection Act). The data provided may be exchanged with other Public Authorities and/or Government Departments as required and permitted by Maltese Law. Transport Malta of Triq Pantar, Lija, Malta LJA2021 is the data controller for the purpose of the privacy laws. The Privacy Notice attached with this application sets out the way in which personal information/data is collected and processed by Transport Malta, as well as the steps that are taken to protect such information.

Data Protection Privacy Notice

Transport Malta of Triq  Pantar, Lija, Malta LJA2021 is the Data Controller for the purpose of the Data Protection Act CAP. 440 and General Data Protection Regulation (EU) (GDPR) 2016/679. This Privacy Notice sets out the way in which we collect and process your Personal Information, as well as the steps we take to protect such information.

- The information we collect and how we use it

  - From this application form Transport Malta collects different types of information which information is that required by Law and is used explicitly for your particular application. It is to be noted that if the required information is not provided the said application cannot be processed.

  - The primary purpose for collecting information is mainly to process the application for the service being applied for, however, your personal information may also be used for related purposes that amongst other include: sending notifications, renewal of licence/certificate after expiry period, and for the provision of information with regards to any legislative amendments which may affect the services offered to you.

- To whom we disclose information

  - This information will be solely used for the reasons detailed above. However there may be cases where personal iinformation is shared with the following third parties for reasons listed below:

- Any third party offering assistance in providing the required service;

- Any law enforcement body who may have any reasonable requirement to access your personal information;

- Third party entities responsible for the data processing contracted by Transport Malta.

- Data Subject Rights

3.1.   With respect to your privacy rights, Transport Malta is obliged to provide you with reasonable access to the Personal Data that you have provided to us. Your other principal rights under data protection law are:

- the right for information;

- the right to access;

- the right to rectification;

- the right to erasure;

- the right to restrict processing;

- the right to object to processing;

- the right to data portability;

- the right to complain to a supervisory authority; and

- the right to withdraw consent.

3.2. 	If you wish to access or amend any Personal Data we hold about you, or to request that we delete any information about you, you may contact us by sending a request to dataprotection.tm@transport.gov.mt. We will acknowledge your request within seventy-two (72) hours and will do our utmost to handle it promptly. We will respond to these requests within a month, with a possibility to extend this period for particularly complex requests in accordance with Applicable Law.

3.3.	At any time, you may object to the processing of your Personal Data, on legitimate grounds, except if otherwise permitted by applicable law.

3.4.	In accordance with Applicable Law, we reserve the right to withhold personal data if disclosing it would adversely affect the rights and freedoms of others. Moreover, we reserve the right to charge a fee for complying with such requests if they are deemed manifestly unfounded or excessive.

- Retention period

  - Personal data will be retained for not more than 3 months from date of application should the application not be submitted complete or is rejected.

  - Once the service related to your application is provided, we will retain your information for as long as needed to provide you with our service, or to comply with our legal obligations, resolve disputes and enforce our agreements.

- Security

5.1. 	We take appropriate security measures to protect against loss, misuse and unauthorized access, alteration, disclosure, or destruction of your information. Additionally, steps will also be taken to ensure the ongoing confidentiality, integrity, availability, and resilience of systems and services processing personal information, and will restore the availability and access to information in a timely manner in the event of a physical or technical incident. All information gathered is kept confidential and is used solely for the purpose indicated herein.

5.2. 	If we learn of a security systems breach, we will inform you of the occurrence of the breach in accordance with applicable law.

- Governing Law

All data collected in this form is processed in accordance with the Privacy Laws that include General Data Protection Regulation (Regulation 2016/679/EU) and Chapter 440 of the Laws of Malta (Data Protection Act).

7. Data Protection Officer

7.1. 	Transport Malta has a Data Protection Officer (“DPO”) who is responsible for matters relating to privacy and data protection. The DPO can be reached at the above address or by email: dataprotection.tm@transport.gov.mt

- Contacting us

8.1. Please address any questions, comments and requests regarding the application process to civil.aviation@transport.gov.mt